AU-11SA-4K22 Update Guide

USER use PC(Mass storage) update F/W:

Update Steps:

- Unplug PSU

- Press and hold the “Power” key, plug in PSU.

- Once entering Update Mode, the front Power LEDs will flash.

- Connect a USB Cable (Mini USB to A-type) between the unit and PC/Laptop

- PC/Laptop will appear up a USB Mass Stroage

- Copy in the file“AU-11SA-4K22_Vx.xx.bin”.

- When updating, the front Power LEDs will flash faster. Once updating process succeeded, unit will reboot automatically.

Note :

If the file is not recongnise by the unit, the front three LEDs will flicker together, just re-plug the power and the USB, and re-do the process.

This update support Win 7 & Win 10 system.